Based on the original works presented in Chickadel et al., (2003)

Elora M H Oades

Department of Civil Engineering

Queen’s University

Canada

Video-Currents-Toolbox V2.0

CIRN

November 2024

# Toolbox Contents

videoCurrentsDemoNew.m

Start here. This demo will walk you through the toolbox and generate figures to visualize results.

vidCurrentsParams.m

The params file, where you can specify details about the input and output data.

See “Quick Start Guide” below.

[sampleStack, timex] = loadVbarRawFile(fileSearchPath, searchDate, transects)

Loads the timex image variables (x, y, Ip) and vbar files (sampleStack) from the folder (fileSearchPath), and which match the searchDate (format: datetime) at any number of desired transects.

[inpDat] = prepDataForInput(sampleStack,params)

This function reformats the vbar data for analysis, according to the params file. For each camera, the data is projected onto a regularly spaced grid and the mid-point of analysis windows are identified (yCenters).

[params] = radonVbarDir(inpDat, params, plotFlag)

With this optional function, the user can automatically identify the dominant direction of the alongshore current, which determines the bounds of the search velocity (vBounds).

[vcTable] = vcTableGen(inputDat, params, xLocation)

This function is used to generate a table for each of the indicated xLocations. This function calls on the original function, videoCurrentGen, to calculate the alongshore velocity of each analysis window.

dataStruct = videoCurrentGen(stack,time,xy,vB,fkB,tWin,tStep,varargin)

# Introduction

Purpose

The Video Currents Toolbox enables the quantification of longshore surf-zone currents from video imagery

Background

Chickadel, C.C., Holman, R.A. and Freilich, M.H., (2003). An optical technique for the measurement of longshore currents. Journal of Geophysical Research: Oceans, 108(C11).

- Uses 1D video data of foam drift to estimate longshore surface currents.

- Converts pixel data into velocity estimates with a model fit for accuracy.

- Provides the foundation of the toolbox, videoCurrentGen.

Oades, E.M., Mulligan, R.P., Palmsten, M.L., Penko, A.M, Elgar, S., Raubenheimer, B. (In-Prep). An improved algorithm for processing optical remote sensing observations of wave-driven alongshore currents informed by non-hydrostatic numerical modelling. In preparation for submission to Journal of Geophysical Research: Oceans.

- Applies the original algorithm along the length of all available pixel transects and filters them using a new quality control metric

- Automatic current direction detection to define velocity search bounds

System Requirements

- MATLAB R2024b

- Optimization Toolbox

- Signal Processing Toolbox

- Image Processing Toolbox

- Statistics and Machine Learning Toolbox

# Input Data

This toolbox is configured to use vbar input files with the following naming convention:

That contains the following variables:

| CAM | 15001 x 1 uint8 | Camera number |
| --- | --- | --- |
| *GAIN | 6 x 2048 double | Camera gain |
| *Name | vbar125 | File name where 125 = x-transect) |
| RAW | 2048 x 15001 uint8 | Pixel intensity (0 – 265) |
| SHUTTER* | 6 x 2048 double | Camera shutter speed at each timestep |
| stats* | 1x1 struct | Collection statistics |
| T | 1 x 2048 | Time in epoch |
| version* | ‘2016-08-19 packed’ | Collection version |
| XYZ | 15001 x 3 double | Pixel position (z = 0) |

*These variables are not required to run videoCurrentsDemoNew

# Output Data

| meanV | OCM guess at the mean current for the timestep |
| --- | --- |
| t | time index for meanV |
| ci | the 95% conf. interval around meanV |
| cispan | the width of ci |
| prob | the probability of the model fit |
| QCspan | the 95th percentile minus the 50th percentile of the timestack histogram, used to measure the amount of video "texture" |
| stdV | the width (std. dev.) of the energy in velocity spectrum |
| vAngle | orientation of the pixel array (radians) |
| SNR | Signal to noise ratio: Afoam / Anoise |
| meanI | Average value of pixel intensity (I) |
| stdI | Standard deviation of the pixel intensity (I) |

# Quick Start Guide

Start by opening the code “videoCurrentsDemoNew.m” & edit the parameters listed in “vidCurrentsParams.m”.

| Variable | Definition | Default Values | Units |
| --- | --- | --- | --- |
| yLims | Alongshore search bounds [yMin yMax] | [0, 1100] | m |
| transects | Cross-shore vbar transect locations | [125, 150, 175, 200, 225] | [m] |
| delY | Resolution of gridded pixels in the alongshore direction | 0.2 | [m] |
| tileSize | Length of analysis window in alongshore-direction | 10 | [m] |
| searchDate | Datetime of search data | 17-Oct 2017 15:59:00 | datetime |
| fSample | Image sampling frequency | 2 | Hz |
| tWindow | The time length of the analysis window | 64 | s |
| tStep | Temporal resolution of the output estimates | 32 | s |
| vBounds | Search limits for alongshore velocity | [vMin vMax] empty | m/s |
| radonCamNum | The camera ID from which the velocity direction (+ or -) will be calculated | 3 | Camera # |
| fkBounds | Frequency, wavenumber bound | [0.015 0.5 0.02 0.5] | Hz, 1/m |

The input parameters are as follows:

# Figures

Figure 1: A sample timex overlayed showing all 5 (five) vbar pixel transects at x = 125, 150, 175, 200, and 225 m.

In the video-currents-Toolbox, camera data are processed separately (regardless of how many cameras exist within the system.

In this figure, the “tileSize” is 20 m, and each point represents the center of the analysis window (tile).

Figure 2: Sample ‘vbar’ timestack from camera 3 over 17-minutes illustrating the oblique lines left from residual foam.

This figure is generated in radonVbarDir when plotFlag = 1.

Figure 3: Sample maxima of the Radon transform applied to the timeseries in Figure 2, which is used to identify the dominant foam tracer direction.

This figure is generated in radonVbarDir when plotFlag = 1.